Red Tail Surveying

Job Workflow Reference

Autonomous Research Pipeline — Team Review Document

GREEN = AI handles automatically     YELLOW = AI assists, human reviews     RED = Human required

# Quick Reference: Job Types

Overview of all job types and their research requirements.

| Code | Type | Adjoiners? | Field Work | Plat? | Duration |
| --- | --- | --- | --- | --- | --- |
| BDY-E | Boundary - Existing Corners | Yes (full) | Monument search | Yes - retracement | 2-4 days |
| BDY-N | Boundary - New Corners | Yes (full) | Set monuments | Yes - original | 3-6 days |
| FT | FEMA Topo | No | Topo shoot | FEMA cert only | 1-2 days |
| SUB | Subdivision | Yes (perimeter) | Set lot corners | Yes - subdivision | 5-10 days |
| LLA | Lot Line Adjustment | Yes (affected) | Set new line | Yes - LLA plat | 2-4 days |
| LC | Land Consolidation | Yes (perimeter) | Verify corners | Yes - consolidation | 2-3 days |
| EAS | Easement | Partial | Locate/set | Yes - exhibit | 1-3 days |
| P | Permit Survey | No | Site verification | Site plan only | 0.5-1 day |
| SE | Survey Exam | Yes (full) | Review only | Report only | 1-2 days |
| ILR | Improvement Location | No | Measure structures | ILR cert | 0.5-1 day |
| WR | Water Rights | No | Ditch mapping | Exhibit map | 1-2 days |

# Workflow 1: BDY-E - Boundary Survey (Existing Corners)

Scenario: Client owns property, corners have been set by a prior survey. Retracing and verifying the existing boundary.

## Research Phase (Office)

| Step | Task | Who | Notes |
| --- | --- | --- | --- |
| 1 | AI - Create project folders & session | AI | Auto folder tree |
| 2 | AI - Search client deed (grantor index) | AI | Auto-select if confidence >= 25 |
| 3 | REVIEW - Verify correct parcel | Human | AI shows top matches, you confirm |
| 4 | AI - Extract legal description | AI | Metes & bounds, lot/block, or TRS |
| 5 | AI - Search client plat (cabinet) | AI | Matches by name in cabinet files |
| 6 | REVIEW - Check plat for prior survey | Human | Look for existing monuments noted |
| 7 | AI - Discover adjoiners (ArcGIS) | AI | 500m radius, touching parcels |
| 8 | AI - Search adjoiner deeds (bulk) | AI | Auto-select best deed per adjoiner |
| 9 | AI - Search adjoiner plats (cabinet) | AI | Cabinet match by last name |
| 10 | REVIEW - Check adjoiner deeds | Human | Check calls for consistency |
| 11 | AI - Build reference table | AI | All deeds & plats cited on plat |
| 12 | AI - Generate DXF boundary | AI | If metes & bounds available |

## Field Phase

| Step | Task | Who | Notes |
| --- | --- | --- | --- |
| 13 | FIELD - Locate existing monuments | Surveyor | Rebar, pipe, stone - per plat |
| 14 | FIELD - Survey control / traverse | Surveyor | Control network for boundary |
| 15 | FIELD - Measure to all corners | Surveyor | Compare to record distances |
| 16 | FIELD - Photograph monuments | Surveyor | Evidence file |
| 17 | FIELD - Resolve discrepancies | PLS | Professional opinion if corners conflict |

## Drafting Phase

| Step | Task | Who | Notes |
| --- | --- | --- | --- |
| 18 | AI+HUMAN - Generate boundary plat | AI+Human | AI creates DXF, human refines |
| 19 | AI - Insert reference table | AI | Auto-generated from research |
| 20 | HUMAN - Surveyors certificate | PLS | Legal certification |
| 21 | HUMAN - Sign & seal | PLS | Licensed surveyor signature |
| 22 | AI - File to project folder | AI | Auto-organize output files |

TIP - Team Customization: Add your specific monument search protocol here. Do you always check PLSS corners first? Do you have a preferred order for checking adjoiners corners?

# Workflow 2: BDY-N - Boundary Survey (New Corners)

Scenario: Property has never been surveyed, or prior survey corners are lost/destroyed. Surveyor must establish new corners.

## Research Phase (Office)

| Step | Task | Who | Notes |
| --- | --- | --- | --- |
| 1-12 | Same as BDY-E above | AI/Human | Full research pipeline |
| 12a | REVIEW - Analyze legal desc for ambiguity | AI+Human | AI flags missing calls, unclosed traverses |
| 12b | HUMAN - Determine corner method | PLS | Proportion, lost/obliterated rules |
| 12c | AI - Check chain of title | AI | Chain search + prior deed analysis |
| 12d | HUMAN - Search for BLM/GLO plats | Human | Federal survey records if PLSS |

## Field Phase

| Step | Task | Who | Notes |
| --- | --- | --- | --- |
| 13 | FIELD - Search for existing evidence | Surveyor | Fences, walls, occupation lines |
| 14 | FIELD - Survey control / traverse | Surveyor | Higher precision - setting corners |
| 15 | FIELD - Calculate new corner positions | PLS | Based on deed, adjoiners, evidence |
| 16 | FIELD - Set rebar & caps at all corners | Surveyor | Monument with PLS number |
| 17 | FIELD - Tie to PLSS or reference | Surveyor | For future recovery |
| 18 | FIELD - Photograph all set monuments | Surveyor | Evidence file |

## Drafting Phase

| Step | Task | Who | Notes |
| --- | --- | --- | --- |
| 19 | AI+HUMAN - Generate boundary plat | AI+Human | Must show NEW corners distinctly |
| 20 | HUMAN - Add monument descriptions | PLS | Set 1/2" rebar with RTS cap |
| 21 | HUMAN - Add basis of bearing | PLS | How corners were established |
| 22 | AI - Insert reference table | AI |  |
| 23 | HUMAN - Certificate (original survey) | PLS | Different language than retracement |
| 24 | HUMAN - Sign & seal | PLS |  |

IMPORTANT: Key difference from BDY-E: The surveyor must establish corners using professional judgment. The AI cannot make this decision - it can only prepare the research.

# Workflow 3: FT - FEMA Topographic Survey

Scenario: Elevation certificate or flood zone determination. No boundary work, no adjoiners.

## Research Phase (Office)

| Step | Task | Who | Notes |
| --- | --- | --- | --- |
| 1 | AI - Create project folders | AI |  |
| 2 | AI - Search client deed | AI | Need property description for cert |
| 3 | REVIEW - Verify correct parcel | Human |  |
| 4 | AI - Pull flood zone data | AI | Query FEMA NFHL layers |
| 5 | SKIP - No adjoiners needed | - | Not needed for topo |

## Field Phase

| Step | Task | Who | Notes |
| --- | --- | --- | --- |
| 6 | FIELD - Set up GPS/total station | Surveyor |  |
| 7 | FIELD - Establish benchmark | Surveyor | NAVD88 datum |
| 8 | FIELD - Shoot topo points | Surveyor | Spot elevations, contours |
| 9 | FIELD - Locate structures | Surveyor | LAG, HAG, next higher floor |
| 10 | FIELD - Photograph reference marks | Surveyor | Required for FEMA form |

## Drafting Phase

| Step | Task | Who | Notes |
| --- | --- | --- | --- |
| 11 | AI - Generate surface / contours | AI (Civil 3D) | TIN surface, 1 ft contours |
| 12 | HUMAN - FEMA Elevation Certificate | PLS | Professional form |
| 13 | HUMAN - Sign & seal | PLS |  |

# Workflow 4: SUB - Subdivision

Scenario: Divide one parcel into multiple lots. Requires perimeter adjoiners, lot layout, and recordable plat.

## Research Phase (Office)

| Step | Task | Who | Notes |
| --- | --- | --- | --- |
| 1 | AI - Create project folders | AI |  |
| 2 | AI - Search parent parcel deed | AI | The deed being subdivided |
| 3 | REVIEW - Check restrictions/easements | Human | Zoning, covenants, setbacks |
| 4 | AI - Search parent plat | AI | Existing plat of parent |
| 5 | AI - Discover perimeter adjoiners | AI | Only surrounding parcels |
| 6 | AI - Bulk research adjoiners | AI | Deeds + plats |
| 7 | HUMAN - Check county regulations | PLS | Lot size mins, road requirements |
| 8 | HUMAN - Design lot layout | PLS/Eng | Road widths, dimensions, utilities |

## Field Phase

| Step | Task | Who | Notes |
| --- | --- | --- | --- |
| 9 | FIELD - Survey parent boundary | Surveyor | Full perimeter |
| 10 | FIELD - Topo of subdivision area | Surveyor | Drainage, utilities, roads |
| 11 | FIELD - Set all new lot corners | Surveyor | Per approved layout |
| 12 | FIELD - Set road/ROW monuments | Surveyor |  |

## Drafting Phase

| Step | Task | Who | Notes |
| --- | --- | --- | --- |
| 13 | AI+HUMAN - Subdivision plat | AI+Human | Lot layout, dimensions, areas |
| 14 | AI - Insert reference table | AI |  |
| 15 | HUMAN - Utility easements/setbacks | PLS/Eng |  |
| 16 | HUMAN - County planning review | PLS | Submit for approval |
| 17 | HUMAN - Certify, sign & seal | PLS |  |
| 18 | HUMAN - Record with county clerk | PLS |  |

# Workflow 5: SE - Survey Exam (Desktop Review)

Scenario: Review existing survey work for accuracy, completeness, or dispute resolution. No new field work.

## Research Phase (Office)

| Step | Task | Who | Notes |
| --- | --- | --- | --- |
| 1 | AI - Create project folders | AI |  |
| 2 | AI - Pull ALL deeds in chain of title | AI | Deep chain search |
| 3 | AI - Pull ALL plats (client + adjoiners) | AI | Full cabinet search |
| 4 | AI - Full adjoiner research | AI | Every neighboring parcel |
| 5 | AI - Analyze descriptions for conflicts | AI | Flag overlaps, gaps |
| 6 | AI - Similarity search on legal desc | AI | Find related parcels via AI |
| 7 | HUMAN - Professional analysis | PLS | Written opinion |
| 8 | HUMAN - Generate written report | PLS | Findings and recommendations |

TIP - Team Customization: Survey Exams are the ideal AI showcase - the entire job is research. The AI can do 80%+ of the work, with the surveyor providing professional analysis.

# Team Discussion Questions

1. Research depth per job type - Are the levels right? Should topo surveys pull any deed research at all?

2. Auto-select confidence - Currently score >= 25 to auto-pick a deed. Too aggressive? Too conservative? Should adjoiners have a lower threshold than the client?

3. Field protocols - What is the standard monument search order? Corner numbering convention? Photo naming?

4. QA gates - Should someone review AI-selected deeds before field day? Or is post-field review sufficient?

5. Deliverables - Standard deliverable sets per job type? (e.g., BDY = plat + legal desc, FT = elevation cert + topo DWG)

6. County specifics - Different counties have different portals and filing requirements. Should the workflow adapt by county?

7. Pricing integration - Should the inquiry form show an automatic estimate based on ML complexity prediction?

8. Client communication - Should the system auto-email the client with a tracking link when research starts?

--- Red Tail Surveying - Taos County, NM ---