Standard Operating Procedure
Deed Research Using the AI Deed & Plat Helper

SOP-RESEARCH-01 | Red Tail Surveying, Inc. | April 2026

Purpose

This SOP documents the standard procedure for conducting deed and plat research using the AI Deed & Plat Helper application. It integrates RTSI's established naming conventions, file structure, and research standards with the app's 6-step workflow to ensure consistent, high-quality research output across all staff.

Prerequisites

You are logged into the AI Deed & Plat Helper (web browser, port 5000)

You have the client name, job number, and job type ready

The job folder has been created in the archive: Z:\0 Archive\{range}\{Job#} {Client}\

The E Research\A Deeds and E Research\B Plats subfolders exist

Job Type Code Reference

| Code | Survey Type |
| --- | --- |
| BDY | Boundary Survey |
| SUB | Subdivision |
| LLA | Lot Line Adjustment |
| FT | Found Corner |
| SE | Section Establishment |
| EAS | Easement |
| P | Plat Only |
| ILR | Improvement Location Report |
| LS | Location Survey |
| WR | Water Rights |
| LC | Land Court |
| BDY-E | Boundary Survey (Easement) |
| BDY-N | Boundary Survey (Non-PLSS) |

Step 1 — Job Setup

1. Open the AI Deed & Plat Helper in your browser and click New Session

2. Enter the client name (format: LAST, FIRST — e.g., GARCIA, JUAN)

3. Enter the RTSI job number (e.g., 3001-BDY)

4. Select the job type from the dropdown (see Job Type Code Reference above)

5. Locate the parcel on the interactive map — zoom in until parcel boundaries are visible, then click the correct parcel

6. Confirm the parcel ID and owner name auto-populate correctly

7. Click Next to proceed to Step 2

Step 2 — Client Deed Research

1. Review the pre-filled search terms and click Search County Records

2. Search priority order: Book and page first (format A149-375, 'contains' search), then Section/Township/Range, then Grantor/Grantee name

3. For TCRS: use 'Other Legal' field with format S#M#T#

4. Click the best matching deed to open the preview

5. Verify the AI-extracted data: Grantor/Grantee names, Book and page, Legal description, Recording date

6. Correct any extraction errors manually and click Accept Deed

7. Save the deed PDF to: E Research\A Deeds\Subject\D1_{ClientLastName}_deed.pdf

8. Mark the file with label: Subject

9. Click Next

Note: If no deed is found for the client, document this in the notes field and flag for supervisor review before proceeding.

Step 3 — Client Plat Research

1. Click Search Cabinet — the app searches all 45,000+ indexed plat documents

2. Also check 1st NM Title (Instrument Type = 'plat', format B-3A for Cabinet B Page 3-A)

3. Additional sources: RTS Database for NM ROW plats; USGLO website for mineral/homestead claims

4. Click any result to preview the plat PDF in the browser

5. When the correct plat is found, click Select

6. Save the plat PDF to: E Research\B Plats\Subject\P1_{ClientLastName}_plat.pdf

7. Mark the file with label: Subject

8. Click Next

Note: After research, plats must be traversed into the DWG on the Plats layer and labeled with the plat source.

Step 4 — Adjoiner Discovery

1. Click Discover Adjoiners — the system runs a GIS spatial query

2. Review the auto-discovered list

3. Also check the deed's metes & bounds description for any 'calls to adjoiners' — add these if missing

4. Remove any obvious errors (roads, ROW strips, parcels not sharing a true boundary)

5. Click Confirm Adjoiners and click Next

Note: The AI uses Taos County ArcGIS parcel data. Unplatted or recently-created parcels may not appear — check manually if the deed calls to an un-indexed neighbor.

Step 5 — Adjoiner Research

1. Click Run Research — the app searches county records and cabinet index for all adjoiners simultaneously

2. Review results for each adjoiner: mark as Complete, Needs Review, or Not Found — add notes for each

3. Pay attention to AI anomaly alerts (yellow/red flags) — document and investigate each one

4. Review any legal description similarity matches — these may be related prior RTSI surveys

5. Save deed PDFs for each adjoiner to: E Research\A Deeds\Adjoiners\D{n}_{Direction}Adj_{LastName}_deed.pdf

6. Save plat PDFs for each adjoiner to: E Research\B Plats\Adjoiners\P{n}_{Direction}Adj_{LastName}_plat.pdf

Step 6 — Package & DXF Export

1. Review the AI completeness score — score must be 80% or higher to proceed

2. If score is below 80%, review flagged gaps before packaging

3. If metes & bounds data is available, click Generate DXF to create the boundary drawing

4. Verify the DXF visually — confirm bearings, distances, and closure look correct

5. Click Save Research Package to finalize the session

6. Export the DXF to: Z:\0 Archive\{range}\{Job#} {Client}\B Drafting\

7. Update the RTSI Reference Table (Excel template at Z:\20 RTSI Workflows\MacLS\Ref Table\)

Direction Label Reference

| Abbreviation | Meaning |
| --- | --- |
| N Adj | North adjoiner |
| S Adj | South adjoiner |
| E Adj | East adjoiner |
| W Adj | West adjoiner |
| NE Adj | Northeast adjoiner |
| NW Adj | Northwest adjoiner |
| SE Adj | Southeast adjoiner |
| SW Adj | Southwest adjoiner |

File Naming Summary

| Document | Folder | Naming Convention |
| --- | --- | --- |
| Client deed | E Research\A Deeds\Subject\ | D1_{LastName}_deed.pdf |
| Client plat | E Research\B Plats\Subject\ | P1_{LastName}_plat.pdf |
| Adjoiner deed | E Research\A Deeds\Adjoiners\ | D{n}_{Dir}Adj_{LastName}_deed.pdf |
| Adjoiner plat | E Research\B Plats\Adjoiners\ | P{n}_{Dir}Adj_{LastName}_plat.pdf |
| Proofing PDF | F PROOFING\ | {Initials} 01 {Job#}-{Type} {Client}.pdf |
| DXF output | B Drafting\ | Standard Civil 3D conventions |

Quality Standards

Every completed research session must have a completeness score of 80% or higher before closing

All AI anomaly alerts must be investigated and documented — do not dismiss without review

Missing deeds or plats for adjoiners must be noted in the session — do not leave blank

The project surveyor or supervisor must sign off on Step 6 before the research package is finalized for any BDY, SUB, or EAS job

SOP-RESEARCH-01 | Red Tail Surveying, Inc. | This is a controlled document. Contact the office administrator to request revisions.